Spaceflight simulator possible user paths

src: /documentation/pages/allScreensPathNames.jpg

Flight physics

During the flight we can control the rocket by:

- Changing rocket angle (α)

- Changing Power for all engines

- Detaching rocket stages.

All flight data is computed by interval time Δt=1 s.

Gravity varies with altitude.

Rocket mass is decreasing while engines are on, because engines burn some fuel during Δt. It is crucial in the simulation.

src: /documentation/physics/physics.jpg

Forces

Fri=Thrusti·cosα-Fgi+Fceni·sinα

Fgi=mi·gi

Thrusti=gi·specificImpulse·massFlowRate·currentPower100

Fceni=Vφi-1·sin(Δφi-1)Δt·mi

Fφi=Fi·sinα

Acceleration and velocity

ari=Frimi

Vri= ari·Δt+Vri-1

aφi=Fφimi

Vφi= aφi·Δt+Vφi-1

Distance and angle

Δhi=Vri·Δt+ari·Δt22

Δli=Vφi·Δt+aφi·Δt22

ΔliR+Δhi =sinΔφi≈Δφi  for small angles

Δφi=sin-1ΔliR+Δhi

φi+1= φi+Δφi

hi+1=hi+Δhi

Determining the position of the rocket in space

All calculations are performed in the polar coordinate system (because it is much easier). So the rocket position is determined by some point O and rocket’s distance from point O (distance = R + altitude in our case), and the angle φ.

We can easily convert these values into Cartesian coordinates

x=R+h·sinφ

y=(R+h)·cosφ

Flight simulator telemetry data

After the flight, three files are generated:

- rocketTelemetry_live.txt

  - this file contains values for all variables during the flight in each time interval.

- rocketTrajectory.png

  - this file show rocket trajectory zoomed into rocket it means that only a part of earth is visible.

- rocketTrajectoryFull.png

  - this file show rocket trajectory in the view of full earth size.

Implementation

src: /documentation/impl/UML.jpg